# 4MP Fixed Bullet Network Camera

EC-B4F28M

## Key Features

- High quality image with 4MP, 1/3"CMOS sensor

- 4MP (2560*1440)@25/20fps; 3MP (2304*1296)@30/25fps; 2MP (1920*1080)@30/25fps; 720P (1280*720)@30/25fps

- Ultra 265, H.265, H.264, MJPEG

- Smart IR, up to 30m (98ft) IR distance

- Supports 128 G Micro SD card

- Support PoE power supply

- 3-Axis

## Specifications

|  | EC-B4F28M |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |  |
| Sensor | 1/3", 4.0 megapixel, progressive scan, CMOS |  |  |  |  |  |
| Minimum Illumination | Colour: 0.01Lux (F2.0, AGC ON) 0Lux with IR on |  |  |  |  |  |
| Day/Night | IR-cut filter with auto switch (ICR) |  |  |  |  |  |
| Shutter | Auto/Manual, 1 ~ 1/100000s |  |  |  |  |  |
| Adjustment angle | Pan: 0° to 360°, Tilt: 0° to 90°, Rotate: 0° to 360° |  |  |  |  |  |
| S/N | >56dB |  |  |  |  |  |
| WDR | DWDR |  |  |  |  |  |
| Lens |  |  |  |  |  |  |
| Lens Type | 2.8mm and 4.0mm @2.0, Fixed-focal |  |  |  |  |  |
| Lens Mount | M12 |  |  |  |  |  |
| Angle of View (H) | 97.0° |  |  | 79.0° |  |  |
| Angle of View (V) | 52.2° |  |  | 42.3° |  |  |
| Angle of View (O) | 107.5° |  |  | 85.1° |  |  |
| DORI |  |  |  |  |  |  |
| DORI Distance | Lens (mm) | Detect (m) | Observe (m) |  | Recognize (m) | Identify (m) |
|  | 2.8 | 63.0 | 25.2 |  | 12.6 | 6.3 |
|  | 4.0 | 90.0 | 36.0 |  | 18.0 | 9.0 |
| Illuminator |  |  |  |  |  |  |
| IR Range | Up to 30m (98ft) IR range |  |  |  |  |  |
| Wavelength | 850nm |  |  |  |  |  |
| IR On/Off Control | Auto/Manual |  |  |  |  |  |
| Video |  |  |  |  |  |  |
| Video Compression | Ultra 265, H.265, H.264, MJPEG |  |  |  |  |  |
| H.264 code profile | Baseline profile, Main profile, High profile |  |  |  |  |  |
| Frame Rate | Main Stream: 4MP (2560*1440), Max 25fps; 3MP (2304*1296), Max 30fps; 1080P (1920*1080), Max 30fps; 720P (1280*720), Max 30fps Sub Stream: D1 (720*576), Max 30fps; 640*360,Max 30fps; 2CIF(704*288), Max 30fps; CIF(352*288), Max 30fps; |  |  |  |  |  |
| Video Bit Rate | 128 Kbps~6144 kbps |  |  |  |  |  |
| U-code | Supported |  |  |  |  |  |
| OSD | Up to 4 OSDs |  |  |  |  |  |
| Privacy Mask | Up to 4 areas |  |  |  |  |  |
| ROI | Up to 8 areas |  |  |  |  |  |
| Motion Detection | Up to 4 areas |  |  |  |  |  |
| Image |  |  |  |  |  |  |
| White Balance | Auto/Outdoor/Fine Tune/Sodium Lamp/Locked/Auto2 |  |  |  |  |  |
| Image Setting | Image rotation, brightness, saturation, contrast, sharpness, gain adjustable by web browser |  |  |  |  |  |
| Digital noise reduction | 2D/3D DNR |  |  |  |  |  |
| Smart IR | Supported |  |  |  |  |  |
| Flip | Normal/Vertical/Horizontal/180° |  |  |  |  |  |
| Dewarping | NA |  |  |  |  |  |
| HLC | Supported |  |  |  |  |  |
| BLC | Supported |  |  |  |  |  |
| Defog | Digital Defog |  |  |  |  |  |
| Events |  |  |  |  |  |  |
| Behavior Detection | Cross line, intrusion(based on human body detection) |  |  |  |  |  |
|  | Motion detection, tampering alarm |  |  |  |  |  |
| General Function | Watermark, IP address filtering, access policy, ARP protection, RTSP authentication, user authentication |  |  |  |  |  |
| Storage |  |  |  |  |  |  |
| Edge Storage | Micro SD, up to 128GB |  |  |  |  |  |
| Network Storage | ANR, NAS(NFS) |  |  |  |  |  |
| Network |  |  |  |  |  |  |
| Protocols | IPv4, IGMP, ICMP, ARP, TCP, UDP, DHCP, RTP, RTSP, RTCP, RTMP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, QoS, SSL/TLS, SNMP |  |  |  |  |  |
| Compatible Integration | ONVIF (Profile S, Profile G, Profile T), API , SDK |  |  |  |  |  |
| User/Host | Up to 32 users. 2 user levels: administrator and common user |  |  |  |  |  |
| Security | Password protection, Strong password, HTTPS encryption, Export operation logs, Basic and digest authentication for RTSP, Digest authentication for HTTP, TLS 1.2, WSSE and digest authentication for ONVIF |  |  |  |  |  |
| Client | EZStation EZView EZLive |  |  |  |  |  |
| Web Browser | Plug-in required live view: IE9+, Chrome 41 and below, Firefox 52 and below |  |  |  |  |  |
|  | Plug-in free live view: Chrome 57.0+, Firefox 58.0+, Edge 16+ |  |  |  |  |  |
| Interface |  |  |  |  |  |  |
| Network | 1 * RJ45 10M/100M Base-TX Ethernet |  |  |  |  |  |
| Certification |  |  |  |  |  |  |
| EMC | FCC: FCC Part 15 |  |  |  |  |  |
| Safety | CE: EN 60950-1 UL: UL60950-1 IEC 60950-1 |  |  |  |  |  |
| General |  |  |  |  |  |  |
| Power | DC 12V±25%, PoE (IEEE 802.3af) |  |  |  |  |  |
|  | Power consumption: Max 5.5W |  |  |  |  |  |
| Power Interface | Ø 5.5mm coaxial power plug |  |  |  |  |  |
| Dimensions (L × W × H) | 165 × 63 × 62mm (6.5” × 2.5” × 2.5”) |  |  |  |  |  |
| Weight | 0.39kg (0.86lb) |  |  |  |  |  |
| Material | Metal |  |  |  |  |  |
| Working Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |
| Storage Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |
| Surge Protection | 4KV |  |  |  |  |  |
| Reset Button | N/A |  |  |  |  |  |
| Ingress Protection | Ingression protection: IP67 |  |  |  |  |  |

### Dimensions

### Accessories

TR-JB03-G-IN     TR-JB07/WM03-G-IN     TR-WM03-D-IN     TR-UM06-E-IN     TR-UP06-IN     TR-A01-IN

TR-SE24-IN                     TR-SE24-A-IN                     TR-CM24-IN

Zhejiang Uniview Technologies Co., Ltd.

Building No.10, Wanlun Science Park, Jiangling Road 88, Binjiang District, Hangzhou, Zhejiang, China (310051)

Email: overseasbusiness@uniview.com; globalsupport@uniview.com

http://www.uniview.com

©2020 Zhejiang Uniview Technologies Co., Ltd. All rights reserved.

*Product specifications and availability are subject to change without notice.