# 4MP HD Intelligent LightHunter IR Fixed Dome Network Camera

IPC324SS-ADF28(40)K-I1

## Key Features

- High quality image with 4MP, 1/3"CMOS sensor

- 4MP (2688*1520)@ 30/25(default)fps; 4MP (2560*1440)@ 30/25fps; 3MP (2304*1296) @ 30/25fps; 2MP (1920*1080) @30/25fps;

- Ultra 265, H.265, H.264, MJPEG

- Intelligent perimeter protection, include cross line, intrusion, enter area, leave area detection

- Based on target classification, smart intrusion prevention significantly reduce false alarm caused by leaves, birds and lights etc., accurately focus on human and, motor vehicle and non-motor vehicle

- Intelligent face capture

- Intelligent People Flow Counting and Crowd Density Monitoring

- Auto Scene Match (ASM)（Only WDR）

- LightHunter technology ensures ultra-high image quality in low illumination environment

- 130dB true WDR technology enables clear image in strong light scene

- Support 9:16 Corridor Mode

- Alarm:1 in/1 out, Audio: 1 in/1 out

- Smart IR, up to 40m (131ft) IR distance

- Supports 256 G Micro SD card

- IK10 vandal resistant and IP67 protection

## Specifications

|  | IPC324SS-ADF28K-I1 |  |  |  | IPC324SS-ADF40K-I1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |  |  |  |  |
| Sensor | 1/3", 4.0 megapixel, progressive scan, CMOS |  |  |  |  |  |  |  |  |
| Minimum Illumination | Colour: 0.002Lux (F1.6, AGC ON) 0 lux with IR |  |  |  |  |  |  |  |  |
| Day/Night | IR-cut filter with auto switch (ICR) |  |  |  |  |  |  |  |  |
| Shutter | Auto/Manual, 1 ~ 1/100000s |  |  |  |  |  |  |  |  |
| Adjustment angle | Pan: 0° ~ 352° |  | Tilt: 0° ~ 70° |  |  |  |  | Rotate: 0° ~ 350° |  |
| WDR | 130dB |  |  |  |  |  |  |  |  |
| S/N | >56dB |  |  |  |  |  |  |  |  |
| Lens |  |  |  |  |  |  |  |  |  |
| Lens | 2.8mm@F1.6 |  |  |  |  | 4.0mm@F1.6 |  |  |  |
| Iris | Fixed |  |  |  |  |  |  |  |  |
| Field of View (H) | 97.68° |  |  |  |  | 81.39° |  |  |  |
| Field of View (V) | 52.67° |  |  |  |  | 43.36° |  |  |  |
| Field of View (D) | 102.52° |  |  |  |  | 86.91° |  |  |  |
| DORI |  |  |  |  |  |  |  |  |  |
| DORI Distance | Lens (mm) | Detect (m) |  | Observe (m) |  |  | Recognize (m) |  | Identify (m) |
|  | 2.8 | 63.0 |  | 25.2 |  |  | 12.6 |  | 6.3 |
|  | 4.0 | 90.0 |  | 36.0 |  |  | 18.0 |  | 9.0 |
| Illuminator |  |  |  |  |  |  |  |  |  |
| IR Range | Up to 40m (131ft) IR range |  |  |  |  |  |  |  |  |
| Wavelength | 850nm |  |  |  |  |  |  |  |  |
| IR On/Off Control | Auto/Manual |  |  |  |  |  |  |  |  |
| Video |  |  |  |  |  |  |  |  |  |
| Video Compression | Ultra 265, H.265, H.264, MJPEG |  |  |  |  |  |  |  |  |
| H.264 code profile | Baseline profile, Main profile, High profile |  |  |  |  |  |  |  |  |
| Frame Rate | Main Stream: 4MP (2688*1520), Max 30fps;4MP (2560*1440), Max 30fps; 3MP (2304*1296), Max 30fps; 1080P (1920*1080), Max 30fps; Sub Stream: 1080P (1920*1080), Max 30fps; 720P (1280*720), Max 30fps; D1 (720*576), Max 30fps; 640*360, Max 30fps; 2CIF(704*288), Max 30fps; CIF(352*288), Max 30fps; Third Stream: D1 (720*576), Max 30fps; 640*360,Max 30fps; 2CIF(704*288), Max 30fps; CIF(352*288), Max 30fps; |  |  |  |  |  |  |  |  |
| Video Bit Rate | 128 Kbps~16 Mbps |  |  |  |  |  |  |  |  |
| U-code | Support |  |  |  |  |  |  |  |  |
| OSD | Up to 8 OSDs |  |  |  |  |  |  |  |  |
| Privacy Mask | Up to 4 areas |  |  |  |  |  |  |  |  |
| ROI | Up to 8 areas |  |  |  |  |  |  |  |  |
| Video Stream | Triple streams |  |  |  |  |  |  |  |  |
| Image |  |  |  |  |  |  |  |  |  |
| White Balance | Auto/Outdoor/Fine Tune/Sodium Lamp/Locked/Auto2 |  |  |  |  |  |  |  |  |
| Digital Noise Reduction | 2D/3D DNR |  |  |  |  |  |  |  |  |
| Smart IR | Support |  |  |  |  |  |  |  |  |
| Flip | Normal/Vertical/Horizontal/180°/90°Clockwise/90°Anti-clockwise |  |  |  |  |  |  |  |  |
| Dewarping | N/A |  |  |  |  |  |  |  |  |
| HLC | Support |  |  |  |  |  |  |  |  |
| BLC | Support |  |  |  |  |  |  |  |  |
| Defog | Digital Defog |  |  |  |  |  |  |  |  |
| Intelligent |  |  |  |  |  |  |  |  |  |
| Smart intrusion prevention | Support false alarm filtering: Cross Line detection, Intrusion detection, Enter Area detection, Leave Area detection |  |  |  |  |  |  |  |  |
| Recognition | Face capture |  |  |  |  |  |  |  |  |
| Statistical Analysis | People counting, support people flow counting and crowd density monitoring |  |  |  |  |  |  |  |  |
| Auto Scene Match (ASM) | Support（Only WDR） |  |  |  |  |  |  |  |  |
| Events |  |  |  |  |  |  |  |  |  |
| Basic Detection | Motion detection, Ultra motion detection, Tampering alarm, Audio detection |  |  |  |  |  |  |  |  |
| General Function | Watermark, IP address filtering, alarm input, alarm output, access policy, ARP Protection, RTSP authentication, user authentication |  |  |  |  |  |  |  |  |
| Audio |  |  |  |  |  |  |  |  |  |
| Audio Compression | G.711U, G.711A |  |  |  |  |  |  |  |  |
| Audio Bitrate | 64 Kbps |  |  |  |  |  |  |  |  |
| Two-way audio | Support |  |  |  |  |  |  |  |  |
| Suppression | Support |  |  |  |  |  |  |  |  |
| Sampling Rate | 8KHZ |  |  |  |  |  |  |  |  |
| Storage |  |  |  |  |  |  |  |  |  |
| Edge Storage | Micro SD, up to 256GB |  |  |  |  |  |  |  |  |
| Network Storage | ANR, NAS(NFS) |  |  |  |  |  |  |  |  |
| Network |  |  |  |  |  |  |  |  |  |
| Protocols | IPv4, IPv6, IGMP, ICMP, ARP, TCP, UDP, DHCP, PPPoE, RTP, RTSP, RTCP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, 802.1x, SNMP, QoS, RTMP,SSL/TLS, SSH |  |  |  |  |  |  |  |  |
| Compatible Integration | ONVIF (Profile S, Profile G, Profile T), API,SDK |  |  |  |  |  |  |  |  |
| User/Host | Up to 32 users. 3 user levels: administrator, common user and operator |  |  |  |  |  |  |  |  |
| Security | Password protection, Strong password, HTTPS encryption, Export operation logs, Basic and digest authentication for RTSP, Digest authentication for HTTP, TLS 1.2, WSSE and digest authentication for ONVIF |  |  |  |  |  |  |  |  |
| Client | EZStation EZView EZLive |  |  |  |  |  |  |  |  |
| Web Browser | Plug-in required live view: IE 10 and above, Chrome 45 and above, Firefox 52 and above, Edge 79 and above |  |  |  |  |  |  |  |  |
|  | Plug-in free live view: Chrome 57.0 and above, Firefox 58.0 and above, Edge 16 and above |  |  |  |  |  |  |  |  |
| Interface |  |  |  |  |  |  |  |  |  |
| Audio I/O | Input: impedance 35kΩ; amplitude 2V [p-p] |  |  |  |  |  |  |  |  |
|  | Output: impedance 600Ω; amplitude 2V [p-p] |  |  |  |  |  |  |  |  |
| Alarm I/O | 1/1 |  |  |  |  |  |  |  |  |
| RS485 | N/A |  |  |  |  |  |  |  |  |
| Built-in Mic | Support |  |  |  |  |  |  |  |  |
| Built-in Speaker | N/A |  |  |  |  |  |  |  |  |
| WIFI | N/A |  |  |  |  |  |  |  |  |
| Network | 1 * RJ45 10M/100M Base-TX Ethernet |  |  |  |  |  |  |  |  |
| Video Output | N/A |  |  |  |  |  |  |  |  |
| Certifications |  |  |  |  |  |  |  |  |  |
| EMC | CE-EMC (EN 55032,EN 61000-3-3,EN IEC 61000-3-2,EN 50130) FCC (FCC 47 CFR part15 B) |  |  |  |  |  |  |  |  |
| Safety | CE-LVD (EN 62368-1) CB (IEC 62368-1) UL/CUL (UL 62368-1, CAN/CSA C22.2 No. 62368-1) |  |  |  |  |  |  |  |  |
| Environment | CE-RoHS (2011/65/EU;(EU)2015/863); WEEE (2012/19/EU) |  |  |  |  |  |  |  |  |
| Protection | IP67 (IEC 60529) IK10 (IEC 62262) |  |  |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |  |  |
| Power | DC 12V±25%, PoE (IEEE 802.3af) |  |  |  |  |  |  |  |  |
|  | Power consumption: 6.5W |  |  |  |  |  |  |  |  |
| Power Interface | Ø 5.5mm coaxial power plug |  |  |  |  |  |  |  |  |
| Dimensions (Ø x H) | Φ111 x 83mm (Ø4.4” x 3.3”) |  |  |  |  |  |  |  |  |
| Weight | 0.42kg (0.93lb) |  |  |  |  |  |  |  |  |
| Material | Metal |  |  |  |  |  |  |  |  |
| Working Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |  |
| Strorage Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |  |
| Surge Protection | 4KV |  |  |  |  |  |  |  |  |
| Reset Button | Support |  |  |  |  |  |  |  |  |
| Web Client Language | 20 Languages Traditional Chinese, English, Simplified Chinese, Polish, German, Russian, French, Korean, Dutch, Czech, Portuguese (Europe), Portuguese (Brazil), Japanese, Thai, Turkish, Spanish (Latin America), Spanish (Europe), Hungarian, Italian, Vietnamese |  |  |  |  |  |  |  |  |

### Dimensions

### Accessories

| TR-JB03-G-IN | TR-JB07/WM03-G-IN | TR-WM03-D-IN | TR-UM06-E-IN |
| --- | --- | --- | --- |
| Junction Box | Wall Mount | Wall Mount | cline Ceiling Mount |
| TR-UP06-IN | TR-A01-IN | TR-SE24-IN | TR-SE24-A-IN |
| Pole Mount | Waterproof Joint | Pendant Mount | Pendant Mount |
| TR-CM24-IN | TR-UC08-C |  |  |
| Pendant Mount | Pole Mount |  |  |

| Junction Mount | Pole Mount |
| --- | --- |
| TR-JB03-G-IN | TR-JB07/WM03-G-IN |
| TR-UM06-E-IN | TR-JB07/WM03-G-IN+TR-UP06-IN |
| TR-CM24-IN+TR-JB03-G-IN | TR-JB07/WM03-G-IN+ TR-UC08-C |

Zhejiang Uniview Technologies Co., Ltd.

Building No. 369, Xietong Road, Xixing Sub-district, Binjiang District, Hangzhou City, Zhejiang Province, China，310051

Email: overseasbusiness@uniview.com; globalsupport@uniview.com

http://www.uniview.com

©2023 Zhejiang Uniview Technologies Co., Ltd. All rights reserved.

*Product specifications and availability are subject to change without notice.