8MP HD ColorHunter With Smart Dual Light  Fixed Turret  Network Camera

E1T-8F28-DL

Features

- High quality image with 8 MP, 1/2.7" CMOS sensor

- 8 MP (3840 × 2160)@20 fps; 5 MP (3072 × 1728)@30/25 fps; 4 MP (2560 × 1440)@30/25 fps; 1080P (1920 × 1080)@30/25 fps

- Ultra 265, H.265, H.264, MJPEG

- 120 dB true WDR technology enables clear image in strong light scene

- Supports 9:16 corridor mode

- Built-in mic

- Smart IR, up to 30 m (98.4 ft.) IR distance

- MicroSD, up to 512 GB

- IP67 protection

- Supports PoE power supply

- 3-axis

Specifications

| Model | E1T-8F28-DL |
| --- | --- |
| Camera |  |
| Max Resolution | 8 MP |
| Sensor | 1/2.7" CMOS |
| Min. Illumination | Color: 0.005 Lux (F1.6, AGC ON), 0 Lux with light |
| Day/Night | IR-cut filter with auto switch (ICR) |
| Shutter | Auto/Manual, 1 ~ 1/100000s |
| Adjustment Angle | Pan: 0° to 360°, Tilt: 0° to 80°, Rotate: 0° to 360° |
| S/N | >56 dB |
| WDR | 120 dB |
| Lens |  |
| Focal Length | 2.8 mm |
| Iris Type | Fixed |
| Iris | F1.6 |
| Field of View (H) | 100.8° |
| Field of View (V) | 55.8 ° |
| Field of View (D) | 105.7° |
| Lens Type | Fixed |
| DORI |  |
| DORI Distance (Lens) | 2.8 mm |
| DORI Distance (Detect) | 84.0 m (275.6 ft.) |
| DORI Distance (Observe) | 33.6 m (110.2 ft.) |
| DORI Distance (Recognize) | 16.8 m (55.1 ft.) |
| DORI Distance (Identify) | 8.4 m (27.6 ft.) |
| Illuminator |  |
| Supplemental Light | Dual light |
| Illumination Distance (IR) | 30 m (98.4 ft.) |
| Illumination Distance (Warm Light) | 30 m (98.4 ft.) |
| Wavelength | 850 nm |
| Light On/Off Control | Auto/Manual |
| Video |  |
| Video Compression | Ultra 265, H.265, H.264, MJPEG |
| H.264 Code Profile | Baseline profile, Main profile, High profile |
| Frame Rate | Main Stream: 8 MP (3840 × 2160), max. 20 fps; 5 MP (3072 × 1728), max. 30 fps; 4 MP (2560 × 1440), max. 30 fps; 1080P (1920 × 1080), max. 30 fps; 720P (1280 × 720), max. 30 fps; Sub Stream: 1080P (1920 × 1080), max. 30 fps; 720P (1280 × 720), max. 30 fps; D1 (720 × 576), max. 30 fps; 640 × 360, max. 30 fps; 2CIF (704 × 288), max. 30 fps; CIF (352 × 288), max. 30 fps; |
| Video Bit Rate | 128 Kbps to 16 Mbps |
| U-code | Support |
| OSD | Up to 4 OSDs |
| Privacy Mask | Up to 4 areas |
| ROI | Support |
| Video Stream | Dual streams |
| Image |  |
| White Balance | Auto, Outdoor, Fine tune, Sodium lamp, Locked, Auto2, Area white balance |
| Digital Noise Reduction | 2D/3D DNR |
| Smart IR | Support |
| Flip | Normal, Flip vertical, Flip horizontal, 180°, 90° Clockwise, 90° Anti-clockwise |
| Dewarping | N/A |
| HLC | Support |
| BLC | Support |
| Defog | Digital defog |
| Image Stabilization | N/A |
| Events |  |
| Basic Detection | Motion detection, Tampering alarm, Audio detection, Human body detection |
| General Function | Watermark, IP address filtering, Access policy, ARP protection, RTSP authentication, User authentication, HTTP authentication |
| Audio |  |
| Audio Compression | G.711U, G.711A |
| Audio Bitrate | 64 Kbps |
| Two-way Audio | N/A |
| Suppression | Support |
| Sampling Rate | 8 kHz |
| Storage |  |
| Edge Storage | MicroSD, up to 512 GB |
| Network Storage | ANR, NAS (NFS) |
| Network |  |
| Protocols | IPv4, IGMP, ICMP, ARP, TCP, UDP, DHCP, RTP, RTSP, RTCP, RTMP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, SSL/TLS, SNMP, QoS, 802.1x |
| Compatible Integration | ONVIF (Profile S, Profile G, Profile T), API, SDK |
| User/Host | Up to 32 users. 3 user levels: administrator, common user and operator |
| Security | Password Protection, Strong Password, HTTPS Encryption, Export Operation Logs, Basic and Digest Authentication for RTSP, Digest Authentication for HTTP, TLS 1.2, WSSE and Digest Authentication for ONVIF |
| Client | UNV-Link, UNV-Link Pro, EZStation |
| Web Browser | Plug-in required live view: IE 10+, Chrome 45+, Firefox 52+, Edge 79+, Plug-in free live view: Chrome 57+, Firefox 58+, Edge 16+ |
| Interface |  |
| Audio I/O | N/A |
| Alarm I/O | N/A |
| Serial Port | N/A |
| Built-in Mic | Support |
| Built-in Speaker | N/A |
| WIFI | N/A |
| Network | 1 × RJ45 10 M/100 M Base-TX Ethernet |
| Video Output | N/A |
| Certification |  |
| EMC | CE-EMC (EN 55032, EN 61000-3-3, EN IEC 61000-3-2, EN 50130-4) FCC (FCC 47 CFR part15 B) |
| Safety | CE LVD (EN 62368-1) UL/CUL (UL 62368-1, CAN/CSA C22.2 No. 62368-1) |
| Environment | CE-RoHS (2011/65/EU;(EU)2015/863) WEEE (2012/19/EU) Packaging and Packaging Waste (94/62/EC) |
| Protection | IP67 (IEC 60529) |
| General |  |
| Power Consumption | Max. 5.3 W |
| Power | DC 12 V±25%, PoE (IEEE 802.3af) |
| Weight | 0.29 kg (0.64 lb.) |
| Power Interface | Ø5.5 mm coaxial power plug |
| Dimensions | Ø126 mm × 93.6 mm (Ø5.0" × 3.7") (Ø × H) |
| Material | Metal + Plastic |
| Working Environment | -30 ℃ to 60 ℃ (-22 ℉ to 140 ℉), Humidity: ≤ 95% RH (non-condensing) |
| Storage Environment | -30 ℃ to 60 ℃ (-22 ℉ to 140 ℉), Humidity: ≤ 95% RH (non-condensing) |
| Surge Protection | 4 KV |
| Reset Button | N/A |
| Web Client Language | 13 Languages: English, Simplified Chinese, Polish, German, Russian, French, Korean, Dutch, Czech, Portuguese, Spanish, Spanish (Europe), Italian |

Dimensions

Accessories

| TR-JB03-H-IN-V2 | TR-JB03-H-IN | TR-JB07/WM03-F-IN-V2 |
| --- | --- | --- |
| Junction Box for Metal Turret Camera(Support wiring from behind) | Junction Box for Metal Turret Camera(Support wiring from behind) | Wall Mounting Assembling Bracket with Back Hole for 3 Inch Hemisphere |
| TR-WM03-B-IN-V2 | TR-UP06-C-IN-V2 | TR-A01-IN |
| Wall Mounting Assembling Bracket for 3 Inch Hemisphere | Pole Mounting Bracket for Bullet Junction Box Only | NPT 3/4'' Waterproof Cable Gland |
| TR-SE24-IN | TR-SE24-A-IN | TR-CM24-IN |
| Dome Pendent Mounting Pole 200mm | Dome Pendent Mounting Pole 500mm | Dome Pendent Mounting Bracket(Indoor) |
| TR-WM03-B-IN | TR-JB07/WM03-F-IN | TR-UC08-C |
| Wall Mounting Assembling Bracket for 3 Inch Hemisphere | Wall Mounting Assembling Bracket with Back Hole for 3 Inch Hemisphere | Bullet&Dome Corner Mounting Bracket |
| TR-UC08-C-V2 |  |  |
| Bullet&Dome Corner Mounting Bracket |  |  |