4MP ColorHunter with Wise-ISP Fixed Warm Light Turret Network Camera

O2T-4F28

Features

- High quality image with 4 MP, 1/1.8" CMOS sensor

- 4 MP (2688 × 1520)@30/25 (default) fps; 4 MP (2560 × 1440)@30/25 fps; 3 MP (2304 × 1296)@30/25 fps; 1080P (1920 × 1080)@30/25 fps

- Ultra 265, H.265, H.264, MJPEG

- ColorHunter with Wise-ISP technology ensures 24-hour full-color images

- Intelligent perimeter protection, including cross line, intrusion, enter area, leave area detection

- Based on target classification, smart intrusion prevention significantly reduces false alarm caused by leaves, birds and lights, accurately focusing on human, motor vehicle and non-motor vehicle

- Intelligent people flow counting and crowd density monitoring

- Smart intrusion prevention with color retrieval

- ASM (auto scene match)

- 130 dB true WDR technology enables clear image in strong light scene

- Supports 9:16 corridor mode

- Alarm I/O: 1/1, audio I/O: 1/1, built-in mic

- White light, up to 30 m (98.4 ft) warm light distance

- MicroSD, up to 512 GB

- Supports NEMA 4X, IK10 vandal resistant and IP67 protection

Specifications

| Model | O2T-4F28 |
| --- | --- |
| Camera |  |
| Max Resolution | 4 MP |
| Sensor | 1/1.8" CMOS |
| Min. Illumination | Color: 0.0003 Lux (F1.0, AGC ON) |
| Shutter | Auto/Manual, 1 to 1/100000 s |
| Adjustment Angle | Pan: 0° to 360°, Tilt: 0° to 75°, Rotate: 0° to 360° |
| S/N | >56 dB |
| WDR | 130 dB |
| Lens |  |
| Focal Length | 2.8 mm |
| Iris Type | Fixed |
| Iris | F1.0 |
| Field of View (H) | 110.4° |
| Field of View (V) | 60.5° |
| Field of View (D) | 115.2° |
| Lens Type | Fixed |
| DORI |  |
| DORI Distance (Lens) | 2.8 mm |
| DORI Distance (Detect) | 43.4 m (142.4 ft) |
| DORI Distance (Observe) | 17.4 m (57.1 ft) |
| DORI Distance (Recognize) | 8.7 m (28.5 ft) |
| DORI Distance (Identify) | 4.3 m (14.1 ft) |
| Illuminator |  |
| Supplemental Light | Warm light |
| Illumination Distance (Warm Light) | 30 m (98.4 ft) |
| Light On/Off Control | Auto/Manual |
| Video |  |
| Video Compression | Ultra 265, H.265, H.264, MJPEG |
| H.264 Code Profile | Baseline profile, Main profile, High profile |
| Frame Rate-Main Stream | 4 MP (2688 × 1520), max. 30 fps; 4 MP (2560 × 1440), max. 30 fps; 3 MP (2304 × 1296), max. 30 fps; 1080P (1920 × 1080), max. 30 fps; 720P (1280 × 720), max. 30 fps |
| Frame Rate-Sub Stream | 1080P (1920 × 1080), max. 30 fps; 720P (1280 × 720), max. 30 fps; D1 (720 × 576), max. 30 fps; 640 × 360, max. 30 fps; 2CIF (704 × 288), max. 30 fps; CIF (352 × 288), max. 30 fps |
| Frame Rate-Third Stream | D1 (720 × 576), max. 30 fps; 640 × 360, max. 30 fps; 2CIF (704 × 288), max. 30 fps; CIF (352 × 288), max. 30 fps |
| Video Bit Rate | 128 Kbps to 16 Mbps |
| U-code | Support |
| OSD | Up to 8 OSDs |
| Privacy Mask | Up to 4 areas |
| ROI | Support |
| Video Stream | Triple streams |
| Image |  |
| White Balance | Auto, Outdoor, Fine tune, Sodium lamp, Locked, Auto2 |
| Digital Noise Reduction | 2D/3D DNR |
| Flip | Normal, Flip vertical, Flip horizontal, 180°, 90° Clockwise, 90° Anti-clockwise |
| Dewarping | N/A |
| HLC | Support |
| BLC | Support |
| Defog | Digital defog |
| Intelligent |  |
| Statistical Analysis | N/A |
| People Counting | Support people flow counting and crowd density detection |
| Attribute Collection | SIP with color retrieval |
| Events |  |
| Basic Detection | Motion detection, Ultra motion detection, Tampering alarm, Audio detection, Object removed, Object Left Behind |
| General Function | Watermark, IP address filtering, Access policy, ARP protection, RTSP authentication, User authentication, HTTP authentication, Alarm input, Alarm output |
| Audio |  |
| Audio Compression | G.711U, G.711A, AAC-LC |
| Audio Bitrate | G.711U/G.711A: 64 kbps, AAC-LC: 128 kbps |
| Two-way Audio | Support |
| Suppression | Support |
| Sampling Rate | G.711U/G.711A: 8 kHz, AAC-LC: 8 kHz, 16 kHz, 48 kHz |
| Storage |  |
| Edge Storage | MicroSD, up to 512 GB |
| Network Storage | ANR, NAS (NFS) |
| Network |  |
| Security | Password Protection, Strong Password, HTTPS Encryption, Export Operation Logs, Basic and Digest Authentication for RTSP, Digest Authentication for HTTP, TLS 1.2, WSSE and Digest Authentication for ONVIF |
| Protocols | IPv4, IPv6, IGMP, ICMP, ARP, TCP, UDP, DHCP, PPPoE, RTP, RTSP, RTCP, RTMP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, 802.1x, SNMP, QoS, SSL/TLS, SSH |
| Compatible Integration | ONVIF (Profile S, Profile G, Profile T, Profile M), API, SDK |
| User/Host | Up to 32 users. 3 user levels: administrator, common user and operator |
| Client | EZStation, APP |
| Web Browser | Plug-in required live view: IE 10+, Chrome 45+, Firefox 52+, Edge 79+, Plug-in free live view: Chrome 57+, Firefox 58+, Edge 16+ |
| Interface |  |
| Audio I/O | 1 Input: impedance 35 kΩ, amplitude 2 V [p-p], 1 Output: impedance 600 Ω, amplitude 2 V [p-p] |
| Alarm I/O | 1/1 |
| Serial Port | N/A |
| Built-in Mic | Support |
| Built-in Speaker | N/A |
| WIFI | N/A |
| Network | 1 × RJ45 10 M/100 M Base-TX Ethernet |
| Video Output | N/A |
| Certification |  |
| EMC | CE-EMC (EN 55032, EN 61000-3-3, EN IEC 61000-3-2, EN 50130) FCC (FCC 47 CFR part15 B) |
| Safety | CE-LVD (EN 62368-1) CB (IEC 62368-1) UL/CUL (UL 62368-1, CAN/CSA C22.2 No. 62368-1) |
| Environment | CE-RoHS (2011/65/EU;(EU)2015/863); WEEE (2012/19/EU) |
| Protection | IP67 (IEC 60529) IK10 (IEC 62262) NEMA 4X |
| General |  |
| Power | DC 12 V±25%, PoE (IEEE 802.3af) |
| Power Consumption | Max. 4.5 W |
| Power Interface | Ø5.5 mm coaxial power plug |
| Dimensions | Ø129 mm × 110 mm (Ø5.1" × 4.3") (Ø × H) |
| Weight | 0.67 kg (1.48 lb) |
| Material | Metal |
| Working Environment | -40 ℃ to 60 ℃ (-40 ℉ to 140 ℉), Humidity: ≤ 95% RH (non-condensing) |
| Storage Environment | -40 ℃ to 60 ℃ (-40 ℉ to 140 ℉), Humidity: ≤ 95% RH (non-condensing) |
| Surge Protection | 4 KV |
| Reset Button | Support |
| Web Client Language | 20 Languages: Traditional Chinese, English, Simplified Chinese, Polish, German, Russian, French, Korean, Dutch, Czech, Portuguese (Europe), Portuguese (Brazil), Japanese, Thai, Turkish, Spanish (Latin America), Spanish (Europe), Hungarian, Italian, Vietnamese |
| Built-in DC12V Output | Support |

Dimensions

Accessories

| TR-JB03-H-IN-V2 | TR-A01-IN | TR-SE24-IN |
| --- | --- | --- |
| Turret Junction Box(Support Wiring From Behind) | NPT 3/4'' Waterproof Cable Gland | Dome Pendent Mounting Pole 200mm |
| TR-SE24-A-IN | TR-CM24-IN | TR-UC08-C-V2 |
| Dome Pendent Mounting Pole 500mm | Dome Pendent Mounting Bracket(Indoor) | Bullet&Dome Corner Mounting Bracket |
| TR-JB07/WM03-F-IN-V2 | TR-WM03-B-IN-V2 | TR-JB03-J-IN |
| Wall Mounting Assembling Bracket with Back Hole For 3 Inches Dome | 3 Inches Dome Wall Mounting Assembling Bracket | VF Turret Junction Box(Support Wiring From Behind) |
| TR-UP06-C-IN-V2 |  |  |
| Pole Mounting Bracket For Bullet Junction Box Only |  |  |